# Marquee Software IOP I2.4 Release Notes

## Compatibility

This software is suitable only for Control Module (CLM), part number 02-2x0336-03P, PCB Number 50-2036-03P (or higher).  This CLM was used for Marquee 8110, 8500 and 9500 series projectors.

This software is compatible with the Marquee main 68k (U35) software V3.2 or higher.

This is also compatible with V3.1 with one exception. The “Remote” jack on the Control Module is not operational.

## About the I/O processor

The Control Module (CLM) of the Marquee series projectors is a complex electronic system. Its correct operation is critical to the performance of a Marquee.

The above mentioned Control Module (CLM)  has a single chip 68HC711E9 micro-controller (U16) that has an on-chip OT PROM (One Time Programmable ROM). The micro-controller handles all serial input and output signals.  U16 is critical for appropriately interpreting the incoming keypad messages and RS232 messages from the ‘RS232 IN’ and ‘RS232 OUT’ connectors.

The programming for U16 is identified by its version number in the form IX.X, for example I2.3.  The intent of the “I” in the version number is to indicate that it is for the I/O (Input/Output) processor.

## About the changes from I2.3 to I2.4

I2.3 is the previous production release.

The following problems with I2.3 have been resolved with version I2.4 (or higher) U16 programming:

- If there is a control system that sends a “key power_on” command via RS232 while the projector is already powered up, then you may have experienced missed messages, projector lockups or other serious misbehavior.

- If the IR keypad was set to protocol A and the Wired Remote keypad was set to Protocol B, the projector would respond to both keypads in either protocol.
Keypad protocol recognition has now been changed to allow the Wired Remote keypad and the IR Sensor keypad protocols to be independently set to A, B, both or none(disabled).

- When the projector was first plugged in or had power restored, a “power on” or “key power_on” command via RS232 would not work. The POWER button on a Marquee or EXTRON switcher did not work either.  After the initial “power on” via keypad, RS232 commands were operational.

If you would like to obtain the U16 I2.4 software, contact an Electrohome Service Depot. Order part number 03-00I240-01P for U16.