# Marquee Software IOP I1.7 Release Notes

## Compatibility

This software is suitable only for Control Module (CLM), part number 02-2x0306-07P (PCB Number 50-2006-02P).  This CLM was used for Marquee 8000 and 9000 series projectors.

This software is compatible with the Marquee main 68k (U35) software V1.18 or higher.

## About the I/O processor

The Control Module (CLM) of the Marquee series projectors is a complex electronic system. Its correct operation is critical to the performance of a Marquee.

The above mentioned Control Module (CLM)  has a single chip 68HC711D3 micro-controller (U16) that has an on-chip OT PROM (One Time Programmable ROM). The microcontroller handles all serial input and output signals.  U16 is critical for appropriately interpreting the incoming keypad messages and RS232 messages from the ‘RS232 IN’ and ‘RS232 OUT’ connectors.

The programming for U16 is identified by its version number in the form IX.X, for example I1.5.  The intent of the “I” in the version number is to indicate that it is for the I/O (Input/Output) processor.

## About the changes from I1.5 to I1.7

I1.5 is the previous production release.

The following problems with I1.5 have been resolved with version I1.7 (or higher) U16 programming:

- If there is a control system that sends a “key power_on” command via RS232 while the projector is already powered up, then you may have experienced missed messages, projector lockups or other serious misbehavior.

- If the Built-In keypad is used at the same time as the IR Sensor and / or Wired Remote keypad then the latter keypads are occasionally locked out until another key on the Built-In keypad is pressed.

If you would like to obtain the U16 I1.7 software, contact an Electrohome Service Depot. Order part number 03-00I170-01P for U16.