Re: Marquee 9000  with Thomson CRT Replacement

Recently, Electrohome has changed the manufacturer of the CRTs used in Marquee 9000 series projectors from Thomson of France to Panasonic. The intent of this bulletin is to inform our customers what to do should CRT replacement be required on units originally equipped with Thomson CRTs.  These projectors are identifible as part number 38-P13091-93.  Electrohome is maintaining stock of Thomson CRT service assemblies. While supplies last, Thomson CRT service assemblies can be ordered with the following part numbers:

03-000242-01P   Red CRT Bulkhead Service Assembly (Thomson)

03-000242-02P   Blue CRT Bulkhead Service Assembly (Thomson)

(yes, the -02 is indeed blue)

03-000242-03P   Green CRT Bulkhead Service Assembly (Thomson)

These service assemblies come with installation instructions for qualified service personnel  and are recommended when replacement of one CRTs is necessary. Should the replacement of two or all three Thomson CRTs be deemed necessary, Electrohome recommends changing the three CRTs to Panasonic.This is the less expensive alternative and also ensures future CRT service assembly availability and avoids having to make the change to Panasonic CRTs at a later date when Thomson CRTs may no longer be available. A mixture of Thomson and Panasonic CRTs on a single projector is not allowed. That is, a Marquee 9000 projector must have either three Panasonic CRTs (and a 34 kV high voltage power supply) or three Thomson CRTs (and a 40 kV high voltage power supply). All of the CRT bulkhead service assemblies for the Marquee 9000 come complete with deflection yokes, convergence coils, magnetic focus coils, etc. They do not come with Video Neck Boards (VNB) or Lenses. In the case of direct Thomson to Thomson replacement, simply reinstall the old VNB(s) and lens(es). If replacing Thomson CRTs with Panasonic CRTs, only the lenses are reused.

If replacing Thomson CRTs with Panasonic, you must order the following parts:

1 -   03-000254-01P Red CRT Bulkhead Service Assembly (Panasonic)

1 -   03-000254-02P Green CRT Bulkhead Service Assembly (Panasonic)

1 -   03-000254-03P Blue CRT Bulkhead Service Assembly (Panasonic)

3 -   03-270300-07P Panasonic VNB (Video Neck Board) Marquee 9000

1 -   03-000234-01P 34 kV HVPS (High Voltage Power Supply)

3 -   39-001596-01P VNB Foam Supports

continued...2

3 -   34-001236-01P FCC Ground Wire

1 -   54-007987-05P Instruction Sheet Thomson To Panasonic 9000 Conversion

The 03-000254-XXP and the 03-270300-07P service assemblies contain instruction sheets that supplement the 54-007987-05P instructions that you must order. As well, we ask that you please return the original 40kV HVPS to Electrohome.