Re: Remove Capacitor on VNBs with HFA1100

Symptom: Marquee 8000 or Marquee 9000 projectors may display uncharacteristically 		      “noisy” images. As well, the image may blank out if the brightness is 		    	       adjusted to 0.

Cause:      U2 on the Video Neck Board (VNB) was changed from a CLC409 to an 	  	      HFA1100. The CLC409 required the presence of C47 a 4.7pF capacitor. The 	  	      HFA1100 does not require the 4.7pF capacitor and may cause the above

symptoms if it is on the VNB.

Action:     If it is felt that the above symptoms are significant, then examine each of the 	 	      three VNBs within the projector and observe whether or not U2 is a CLC409 	 	      or an HFA1100. If U2 is an HFA1100, then remove C47 if it is present on the  	      board. Consult the service manual for your particular model of projector to 	 	      assist you with any of the projector dissassembly required in order to be able to  	      observe the VNB components. The U2 and C47 component designations apply 	      to all the Marquee models (8000, 9000 with Thomson CRTs and 9000 with 	  	      Panasonic CRTs).