Re: Marquee Focus Module Reliability Improvement

Symptoms: The projector does not power-up and the +15V LED or the -15V LED (or 		         both) on the Low Voltage Power Supply (LVPS) come on. These LEDs are

visible through the grillwork on the lower front panel of the projector just

below the lenses. The -15V LED is first in the row of seven power supply 	    	        diagnostic LEDs (on the left). The +15V LED is last in the row of seven

LEDs (on the right).  If the projector does power-up then the static electrical

focus adjustment on the red or all three colours may not work. This is

easily tested by adjusting the master FOCUS in the PICTURE menu and

observing the focus of the image one colour at at time (don’t forget to return

the adjustment to 50). The test can also be done in the service menu under

RGB FOCUS and applies to the operation of the adjustment at the centre of

the screen only.

Cause:         The above symptoms are attributable to the failure of an IC on the focus

module (FCM). The IC in question is a TLO84 device used in the U101,

U201 or U301 positions. If U101 fails, then only the red static electrical

focus should be affected. If U201 or U301 fail then the operation of the static

electrical focus on all three colours will be affected. The failed TLO84 device

is often visibly blown, either a piece will be broken off of the IC case or a

dark spot will appear near pins 4 or 11. TLO84 devices were used only

on FCMs with circuit boards labelled with the number 50-2004-03P.

The TLO84 devices have since been replaced with MC34074 parts which are

a direct but more robust replacement. Earlier versions of the FCM with

circuit boards labelled 50-2004-01P used devices from the MC3407X family

and were immune to this problem. There were no 50-2004-02P labelled

circuit boards released.

Action:         If you have observed the above symptoms on your Electrohome

Marquee 8000 or Marquee 9000 series projector and have a FCM with a

circuit board labelled 50-2004-03P which contains TLO84 devices in the

U101, U201 and U301 positions, then qualified service personnel are

welcome to attempt repair of the module by replacing U101, U201 and

U301 with MC34074 parts only (available from Motorola Semiconductor

Products). Remember to use appropriate static sensitive device precautions

when handling Marquee modules. Another course of action available is to

return the defective Marquee Focus Module (FCM) under warranty to an

continued...2

authorized Electrohome service depot. A replacement module can be

ordered with the following service module part number.

Service Assembly Focus Module (FCM) .......... 03-270304-03P.

The projector will require a full electrical focus alignment after replacing or

repairing the focus module. See your Marquee service and user’s manuals

for details.