Re: Marquee Switcher/Projector Communications

If you’ve been having problems getting your Marquee Signal Switcher to communicate with your Marquee series projector then we would like to suggest the following.

1. Turn off the projector with either the built-in or Infrared (IR) keypad. If 	necessary, unplug and re-connect the line cord to the Low Voltage Power

Supply (LVPS) on the projector. Only the amber STANDBY LED should be

illuminated on the control module panel (beside the green POWER LED).

2. Establish the RS-232 connection between the port labelled “TO SWITCHER”

on the projector and the port labelled “RS232 OUT” on the switcher. See the

Marquee Signal Switcher Manual (part number 54-7000-01P) appendix A for 	details on the construction of the RS-232 cable. The manual (provided with the

switcher) also provides details on multi-switcher systems and on using the switcher

without a Marquee series projector (called Stand-Alone operation).

3. Make sure that the switcher line cord is plugged in (at this point, the switcher

can be either on or off and in either normal or stand-alone mode). Turn on the

projector from the built-in or IR keypad. This should turn on the green switcher

POWER button LED and properly re-establish switcher to projector

communications. You need to turn on the system from the projector because it

is the master in a pseudo master-slave system where the switcher is the slave.

Turning on the system from the switcher (the slave) will not necessarily clear up

the communications problems. In fact, if the switcher is in stand-alone mode, then

communications with the projector are disabled until the projector re-establishes 	communications. In the stand-alone mode, the amber indicator above the

STANDBY push-button is not illuminated. It is illuminated in normal mode.

Communications may need to be re-established in either mode of switcher

operation.

4. If the projector displays a “SWITCHER NOT CONNECTED” message when 	the source is switched to a switcher input, then recheck the RS-232 cable and 	connections described in step 2 above. Retry step 3 if neccessary.