Re: Bright Vertical Line on Right Edge Of Image

Applies to:  Marquee 8000 and 9000 series projectors.

Symptoms: The visible projected image has a bright narrow vertical line on the right edge 	        of the screen. The presence of this problem can be confirmed by adjusting the

RIGHT BLANKING in the PICTURE menu. The line should move as the 	     	        adjustment is made. A fainter narrow vertical line on the left edge of the 	 	        visible image may also be seen. It should be observed to move as LEFT

BLANKING is adjusted.

Cause:        The signals that cause the image to go black during horizontal retrace are 	   	        slightly mis-timed such that the projector switches to an internally generated 		        video image just before the image actually goes to black. Appropriately

adding resistors to the relevant circuitry can correct the timing of the signals.

Action:       Qualified service personnel are welcome to perform the following

modification to the Video Input Module (VIM). This applies to printed  	 	        circuit boards labelled 50-2005-03P only.

1. Cut the circuit board trace between R165 and U8 pin 12.

2. Cut the trace between R166 and U8 pin 3. (runs under U8)

3. Add a 1 kohm resistor from the pad of R165 closest to U8 to U8 pin 12 	  	       (effectively across the cut made in step 1 above). See the drawing on

page 2. It is alright to use a standard 1 kohm resistor. It may be preferable 		        however to order resistors from Electrohome since standard resistors are 	  	        larger than those obtainable from Electrohome and may prove difficult to 	 	        install with the surface mount components on the VIM. The resistors can be 	 	        ordered from Electrohome Service Depots by asking for part number

80-110015-11P.

4. Add a 1 kohm resistor from the pad of R166 closest to U8 to U8 pin 3

(effectively across the cut made in step 2 above). See the drawing on

page 2.

continued...2

Use the following drawing as a guideline when modifying the VIM (labelled 50-2005-03P only).

U44

add 1 kohm resistor

(80-110015-11P)

cut trace here

cut trace here

add 1 kohm resistor

(80-110015-11P)

R166

R165

U8

pin 3